- VISUAL NEAR INFRARED (VNIR 3N) COMPOSITE

- Land Processes Distributed Active Archive Center (LP DAAC)

- (Included in the ASTER 14DMO Product)

## Summary

The Visual Near Infrared band is part of the ASTER On-Demand L3 DEM and Orthorectified Images multi-file product that contains both the Digital Elevation Model (DEM), and the Orthorectified Image products. Each product delivery includes one DEM data file, and fifteen orthorectified data files in GeoTIFF format. This reads me file describes the information used to create a composite 3N band.

The ASTER On-Demand Orthorectified Image products contain imagery transformed from a perspective projection to an orthogonal one. An orthorectified image possesses the geometric characteristics of a map, with near-vertical views for every location. These products are terrain-corrected, provide radiometrically calibrated radiance, and are mapped to the Universal Transverse Mercator coordinate system. The inputs include the following: an ASTER Level-1A Reconstructed Unprocessed Instrument data set; georeferencing information from the ASTER instrument's and Terra platform's ephemeris and attitude data. The VNIR output product includes four orthorectified images, as listed below.

ASTER Orthorectified Product Output Information

| Spectral Range (µm) | Units | Data Type | Valid Range | Unit Conversion Coefficient* |
| --- | --- | --- | --- | --- |
| VNIR (15 Meters) |  |  |  | Normal Gain |
| Band 1 (0.52 - 0.60) | w/m2/sr/µm | 8-bit unsigned integer | 0 - 255 | 1.688 |
| Band 2 (0.63 - 0.69) | w/m2/sr/µm | 8-bit unsigned integer | 0 - 255 | 1.415 |
| Band 3N (0.78 - 0.86) | w/m2/sr/µm | 8-bit unsigned integer | 0 - 255 | 0.862 |
| Band 3B (0.78 - 0.86) | w/m2/sr/µm | 8-bit unsigned integer | 0 - 255 | 0.862 |

## Citing These Data:

## These data are distributed by the Land Processes Distributed Active Archive Center (LP DAAC), located at the U.S. Geological Survey (USGS) Earth Resources Observation and Science Center (EROS) http://LPDAAC.usgs.gov.

## Overview Table

| Vgroup Data Fields/ spectral range (µm) | Units | Data Type | Valid range | Cross-track pointing capability |
| --- | --- | --- | --- | --- |
| Band 3N (0.78 - 0.86) | Digital Counts | 8-bit unsigned integer | 0 - 255 | +/- 24° |

## Table of Contents

1. Contacts and Acknowledgments

2. Detailed Data Description

3. Data Access and Tools

4. Document Information

## 1. Contacts and Acknowledgments:

### Contacts:

### LP DAAC User Services

### U.S. Geological Survey (USGS)

### Earth Resources Observation and Science Center (EROS)

### 47914 252nd Street

### Sioux Falls, SD 57198-0001

### Phone: 605-594-6116 Toll Free: 866-573-3222

### 866-LPE-DAAC Fax: 605-594-6963 Email: LPDAAC@eos.nasa.gov Web: http://LPDAAC.usgs.gov

### Acknowledgements:

Several graduate students and volunteers worked to process the original ASTER data. We would also like to thank Michigan Technological University Remote Sensing Institute (http://www.rsi.mtu.edu/) for acquiring the images from LPDAAC.

## 2. Data Description:

### Format: ESRI ASCII Grid format converted from original EDG Data.

Spatial Extension of composite: From coordinates in decimal degrees WGS 1984 (-111.73, 28.01) to (-109.31, 31.31). This file is for the area in the Sierra Los Locos basin only.

### Resolution: 15 meters

### Composite File Data Containing:

The composite file contains data from 25 individual images that were purchased by the Remote Sensing Institute at Michigan Technological University. Below is the list of individual images used for creating the composite Sonora River DEM. All of the images were taken by day.

| NAME | DATE | TIME |
| --- | --- | --- |
| AST14DMO_00303312007181638_20080601142245_4537 | 31/03/2007 | 18:16:38 |
| AST14DMO_00303312007181656_20080601142305_4581 | 31/03/2007 | 18:16:56 |
| AST14DMO_00305272007181007_20080601142305_4593 | 27/05/2007 | 18:10:07 |
| AST14DMO_00304252007181017_20080601142304_4596 | 25/04/2007 | 18:10:17 |
| AST14DMO_00303312007181647_20080601142305_4597 | 31/03/2007 | 18:16:47 |
| AST14DMO_00304252007181026_20080601142304_4598 | 25/04/2007 | 18:10:26 |
| AST14DMO_00310092004180850_20080430154521_4598 | 09/10/2004 | 18:08:50 |
| AST14DMO_00308282006180931_20080430154521_4624 | 28/08/2006 | 18:09:31 |
| AST14DMO_00311162006180916_20080430154520_4632 | 16/11/2006 | 18:09:16 |
| AST14DMO_00307142007181021_20080601142315_4697 | 14/07/2007 | 18:10:21 |
| AST14DMO_00307142007181030_20080601142315_4698 | 14/07/2007 | 18:10:30 |
| AST14DMO_00302042007181030_20080601142315_4700 | 04/02/2007 | 18:10:30 |
| AST14DMO_00302042007181013_20080430154540_5414 | 04/02/2007 | 18:10:13 |
| AST14DMO_00302042007181021_20080430154540_5416 | 04/02/2007 | 18:10:21 |
| AST14DMO_00303312007181620_20080430154550_5494 | 31/03/2007 | 18:16:20 |
| AST14DMO_00303312007181629_20080430154550_5499 | 31/03/2007 | 18:16:29 |
| AST14DMO_00303312007181638_20080430154550_5504 | 31/03/2007 | 18:16:38 |
| AST14DMO_00304252007180959_20080430154550_5507 | 25/04/2007 | 18:09:59 |
| AST14DMO_00304252007180951_20080430154600_5547 | 25/04/2007 | 18:09:51 |
| AST14DMO_00304252007181008_20080430154600_5552 | 25/04/2007 | 18:10:08 |
| AST14DMO_00304252007181017_20080430154600_5563 | 25/04/2007 | 18:10:17 |
| AST14DMO_00307142007181012_20080430154611_5758 | 14/07/2007 | 18:10:12 |
| AST14DMO_00304062006180933_20080430154621_6498 | 06/04/2006 | 18:09:33 |
| AST14DMO_00304062006180925_20080430154621_6502 | 06/04/2006 | 18:09:25 |
| AST14DMO_00304132006181519_20080430154636_6606 | 13/04/2006 | 18:15:19 |

## File Size: 1802592 KB

## 3. Data Access and Tools:

### Data Access:

Data are available via direct download from IRES webpage http://www.ees.nmt.edu/vivoni/sonora/www/.

### Software and Tools:

A GIS software is required to view these data (i.e. ESRI ArcGIS)

## 4. Document Information:

### Document Creation Date: June 24, 2008