- Soil moisture and soil temperature data from Rayon tower sampling plots

- Sonora IRES Field Campaign 2006-2008

## Summary

These data have being collected during the Sonora IRES (International Research Experience for Students) Field Campaign 2006-2008 by New Mexico Institute of Mining & Technology associated with Universidad de Sonora, Instituto Tecnológico de Sonora, University of Arizona, and University of New Mexico.

The purpose of these data is to study the variability of soil moisture and soil temperature within the 250-m by 250-m area around the flux tower, located in Rayon, Sonora Mexico. There are 29 sampling plots whose dimensions are 1-m x 1-m. The measurements are taken at each corner of the square (4) and at the center (1), five measurements per plot.

The data is taken daily during the Sonora field campaigns in the 2006, 2007 and 2008 summer seasons. Soil moisture is measured with a theta probe type ML2 from Delta T Devices. Soil temperature is measured with a thermometer at three depths: 1-cm, 5-cm, and 10-cm. Additionally, an infrared thermometer for surface temperature is used.

## Overview Table

| Category | Description |
| --- | --- |
| Data format | Excel Spreadsheets: TowerDaily2006.xls; TowerDaily2007.xls; TowerDaily2008.xls |
| File naming convention | Files are named according to the year of the measurements. Each sheet in the files contains one (1) sampling day. |
| File size | 480 KB to 640 KB |
| Parameter(s) | Soil temperature, soil moisture |
| Procedures for obtaining data | Direct download and Email. |

## Citing These Data:

## These data are distributed by the Surface Hydrology Research Group of New Mexico Institute of Mining and Technology. http://www.ees.nmt.edu/vivoni/sonora/www/

## Table of Contents

1. Contacts and Acknowledgments

2. Data Description

3. Data Access and Tools

4. Document Information

## 1. Contacts and Acknowledgments:

### Technical Contact:

Dr. Enrique R. Vivoni

Associate Professor of Hydrology

Research Hydrologist

Department of Earth & Environmental Science

New Mexico Tech, Socorro, NM 87801, MSEC 244

Tel: 575 835-5611

Email: vivoni@nmt.edu

http://www.ees.nmt.edu/vivoni/

### Acknowledgements:

Several graduate students and volunteers worked to process the field data. We would also like to thank all the participants of the Sonora field campaign.

## 2. Data Description:

### Format:

Excel spreadsheets contain the relevant data sets. Each sheet has a table with site locations and variables There are 3 files.

### File Data Containing:

The files contain data from Rayon tower plots of soil moisture and soil temperature. Each sheet in each file contains data from all the plots for the specific date of measurements.

### File Size:

File sizes range from 5 KB to 12 KB.

#### File Parameters:

| Column Number | Name | Description |
| --- | --- | --- |
| Column 1 | ID | Plots ID with matrix convention. Example: ID 1,2: Column 1 - Row 2. |
| Column 2 | TIME | Time when variables were collected |
| Column 3 | ST_A_IRT | Soil surface temperature ( C) at corner A of the square plot. |
| Column 4 | ST_A_1cm | Soil temperature ( C) at corner A of the square plot. Depth: 1 cm |
| Column 5 | ST_A_5cm | Soil temperature ( C) at corner A of the square plot. Depth: 5 cm |
| Column 6 | ST_A_10cm | Soil temperature ( C) at the corner A of the square plot. Depth: 10 cm |
| Column 7 | ST_B_IRT | Surface soil temperature ( C) at the corner B of the square plot. |
| Column 8 | ST_B_1cm | Soil temperature ( C) at the corner B of the square plot. Depth: 1 cm |
| Column 9 | ST_B_5cm | Soil temperature ( C) at the corner B of the square plot. Depth: 5 cm |
| Column 10 | ST_B_10cm | Soil temperature ( C) at the corner B of the square plot. Depth: 10 cm |
| Column 11 | ST_C_IRT | Surface soil temperature ( C) at the corner C of the square plot. |
| Column 12 | ST_C_1cm | Soil temperature ( C) at the corner C of the square plot. Depth: 1 cm |
| Column 13 | ST_C_5cm | Soil temperature ( C) at the corner C of the square plot. Depth: 5 cm |
| Column 14 | ST_C_10cm | Soil temperature ( C) at the corner C of the square plot. Depth: 10 cm |
| Column 15 | ST_D_IRT | Surface soil temperature ( C) at the corner D of the square plot. |
| Column 16 | ST_D_1cm | Soil temperature ( C) at the corner D of the square plot. Depth: 1 cm |
| Column 17 | ST_D_5cm | Soil temperature ( C) at the corner D of the square plot. Depth: 5 cm |
| Column 18 | ST_D_10cm | Soil temperature ( C) at the corner D of the square plot. Depth: 10 cm |
| Column 19 | ST_E_IRT | Surface soil temperature ( C) at the corner E of the square plot. |
| Column 20 | ST_E_1cm | Soil temperature ( C) at the corner E of the square plot. Depth: 1 cm |
| Column 21 | ST_E_5cm | Soil temperature ( C) at the center of the square plot. Depth: 5 cm |
| Column 22 | ST_E_10cm | Soil temperature ( C) at the center of the square plot. Depth: 10 cm |
| Column 23 | SM_A_per | Vol. Soil moisture (%) at corner A of the square plot. |
| Column 24 | SM_A_mV | Vol. Soil moisture (mV) at corner A of the square plot. |
| Column 25 | SM_B_per | Vol. Soil moisture (%) at corner B of the square plot. |
| Column 26 | SM_B_mV | Vol. Soil moisture (mV) at corner B of the square plot. |
| Column 27 | SM_C_per | Vol. Soil moisture (%) at corner C of the square plot. |
| Column 28 | SM_C_mV | Vol. Soil moisture (mV) at corner C of the square plot. |
| Column 29 | SM_D_per | Vol. Soil moisture (%) at corner D of the square plot. |
| Column 30 | SM_D_mV | Vol. Soil moisture (mV) at corner D of the square plot. |
| Column 31 | SM_E_per | Vol. Soil moisture (%) at the center of the square plot. |
| Column 32 | SM_E_mV | Vol. Soil moisture (mV) at the center of the square plot. |

Additional information contained in the files includes: (a) plot mean and standard deviation for soil moisture and 1-cm soil temperature; (b) footprint mean and standard deviation for soil moisture and 1-cm soil temperature and (c) corrected soil moisture values using factory specific calibration.

NOTE: The corrected soil moisture values should always be used.

The next figure specifies the location of samples A-E in a plot:

| A E | B |
| --- | --- |
| C | D |

### Spatial Location: (UTM Zone 12, WGS 1984)

| ID | Northing (m) | Easting (m) | Elevation (m) |
| --- | --- | --- | --- |
| 5,5 | 3290277 | 544930 | 622 |
| 4,5 | 3290274 | 544878 | 626 |
| 3,5 | 3290288 | 544808 | 626 |
| 2,5 | 3290293 | 544758 | 631 |
| 1,5 | 3290302 | 544697 | 627 |
| 1,4 | 3290238 | 544694 | 633 |
| 2,4 | 3290235 | 544750 | 633 |
| 3,4 | 3290239 | 544804 | 637 |
| 4,4 | 3290239 | 544869 | 638 |
| 5,4 | 3290228 | 544949 | 637 |
| 5,3 | 3290179 | 544939 | 641 |
| 4,3 | 3290184 | 544869 | 638 |
| 3,3 | 3290185 | 544816 | 641 |
| 2,3 | 3290185 | 544752 | 638 |
| 1,3 | 3290183 | 544688 | 637 |
| 1,2 | 3290110 | 544679 | 631 |
| 2,2 | 3290118 | 544758 | 633 |
| 3,2 | 3290126 | 544823 | 633 |
| 4,2 | 3290111 | 544875 | 633 |
| 5,2 | 3290117 | 544926 | 640 |
| 5,1 | 3290062 | 544932 | 641 |
| 4,1 | 3290052 | 544873 | 639 |
| 3,1 | 3290059 | 544802 | 628 |
| 2,1 | 3290061 | 544752 | 623 |
| 1,1 | 3290051 | 544683 | 629 |
| A | 3290159 | 544811 | 634 |
| B | 3290181 | 544850 | 636 |
| C | 3290204 | 544812 | 637 |
| D | 3290179 | 544785 | 636 |

## 3. Data Access and Tools:

### Data Access:

### Data Access:

Data are available via direct download from Sonora IRES Field Campaigns webpage http://www.ees.nmt.edu/vivoni/sonora/www/. This website may change location.

### Software and Tools:

No special tools are required to view these data. A spreadsheet program, which recognizes tab-delimited text files, such as MS Excel is recommended.

## 4. Document Information:

The value NA has been used to replace empty space and no data in the text files. Document creation date on 6 February 2009.

Use of the data should be accompanied by acknowledgments, including citation to the following paper. Check for final publication details of the submitted paper:

Vivoni, E.R., Watts, C.J., Rodriguez, J.C., Garatuza-Payan, J., Mendez-Barroso, J.C. and Saiz-Hernandez, J. 2009. Improved Land-Atmosphere Relations through Distributed Footprint Sampling in a Subtropical Scrubland during the North American Monsoon. Journal of Arid Environments (In Review).